Tremco, Inc. Commercial Sealants & Waterproofing

Section 07 18 00 TRAFFIC COATINGS Vehicular Guide Specification

Specifier: This guide specification section specifies Tremco Vulkem® 350NF/Vulkem Epoxy WC/[950NF]951NF and 360NF/Epoxy WC/[950NF]951NF low-VOC deck coating systems of polyurethane traffic coating products.

This deck coating system is a cold applied, vehicular traffic deck coating system designed for waterproofing concrete slabs and protecting occupied areas underneath from water damage. Additionally, the system will protect concrete from damaging effects of deicing salts, gasoline, oils and antifreeze.

Vulkem 950NF and Vulkem 951NF topcoats are high-performance, neighbor-friendly, two-part polyurethane coatings for vehicular applications where low odor and extremely durable coatings are desired. It is a nearly 100 percent solids, VOC-compliant system that is extremely durable and can be installed in a fast-track method. Vulkem 950NF topcoat can be used as a topcoat for interior applications. Vulkem 951NF is UV stable for exterior applications.

Vulkem Epoxy WC is a two-part, (1:1) high solids, low odor, VOC compliant, epoxy wear coat applied after the Vulkem Base Coat has cured. The intermediate coat is loaded with aggregate to give the system excellent impact and abrasion resistance.

Vulkem 350NF Base Coat is a single-component, low-odor, low-VOC, polyurethane membrane that bonds firmly to clean, dry concrete and metal. It retains its integrity even if substrate movement causes hair-line cracks of up to 1/16 in. (1.5mm). Vulkem 350NF is available in roller (R) and in self-leveling (SL) grade for vertical and horizontal application.

Vulkem 360NF Base Coat is a neighbor-friendly, low-odor, VOC-compliant, water-cured, rapid-setting polyurethane basecoat that possesses tenacious adhesion primarily to clean and dry concrete. Vulkem 360NF is a versatile basecoat that can be applied at various thicknesses, making it ideal for patching and sloping areas on restoration jobs. In areas not exposed to vehicular wheel traffic.  Use a cementitious sloping, patching , and repair mortar when exposed to vehicular traffic.

Or

Vulkem 350[350NF]/Vulkem Epoxy WC/346TC standard deck coating system of polyurethane traffic coating products.

This deck coating system is a cold applied vehicular traffic deck coating system designed for waterproofing concrete slabs and protecting occupied areas underneath from water damage. Additionally, the system will protect concrete from damaging effects of deicing salts, gasoline, oils and antifreeze.

Vulkem 346TC Top Coat is an aliphatic one-component polyurethane that is applied after the Vulkem Epoxy WC intermediate coat has cured. Interlaminar adhesion to Vulkem Epoxy WC is exceedingly strong. The top coat affords excellent abrasion resistance, UV stability and chemical resistance to complete this Vulkem Traffic Deck Coating System.

Vulkem 350 Base Coat is a single-component urethane membrane that bonds firmly to clean, dry concrete and metal. It retains its integrity even if substrate movement causes hair-line cracks of up to 1/16" (1.5 mm). If cut or damaged, Vulkem 350 will prevent water migration between itself and the substrate. Vulkem 350 is available in roller (R) and in self-leveling (SL) grade for vertical and horizontal application.

Basic Uses

Specially formulated for the odor-sensitive applications in heavily occupied areas It is ideal for restoration work where both time and odor are a concern.

Medium-Duty applications include waterproofing parking stalls and similar primarily concrete substrates requiring an elastomeric waterproofing system.

Heavy-Duty vehicular traffic deck coating applications include waterproofing concrete slabs and protecting occupied areas underneath from water damage. The system also protects concrete from damaging effects of water deicing salts, chemicals, gasoline, oils and anti-freeze.

This section is easily edited using several common commercial specification software tools.

We recommend you consult with your Tremco construction technical representative, who can be contacted through: Tremco, Inc., Commercial Sealants and Waterproofing Division, Beachwood OH; (866) 321-6357; email: techresources@tremcoinc.com; www.tremcosealants.com.

Tremco products appear in the following CSI MasterFormat guide specifications available from Tremco:

• Section 07 01 91 Joint Sealant Rehabilitation and Replacement

• Section 07 14 13.01 Hot Fluid-Applied Waterproofing, Deck (TREMproof 6100)

• Section 07 14 16.01 Cold Fluid-Applied Waterproofing, Vertical and Deck (TREMproof 250GC)

• Section 07 14 16.02 Cold Fluid-Applied Waterproofing, Vertical (TREMproof 250GC)

• Section 07 14 16.03 Cold Fluid-Applied Waterproofing, Deck (TREMproof 250GC)

• Section 07 17 16.01 Bentonite Waterproofing (Paraseal)

• Section 07 17 16.02 Bentonite Waterproofing (Paraseal GM/LG 60 mil)

• Section 07 18 00.01 Traffic Coatings, Vehicular

• Section 07 18 00.02 Traffic Coatings, Pedestrian

• Section 07 27 13 Modified Bituminous Sheet Air Barriers, Vapor-Retarding (ExoAir 110)

• Section 07 27 23 Board Product Air Barriers, Vapor Permeable (SECUREROCK ExoAir 230)

• Section 07 27 26.01 Fluid-Applied Membrane Air Barriers, Vapor-Retarding (ExoAir 120)

• Section 07 27 26.02 Fluid-Applied Membrane Air Barriers, Vapor Permeable (ExoAir 220)

• Section 07 27 26.03 Fluid-Applied Membrane Air Barriers, Vapor Permeable (ExoAir 230)

• Section 07 92 00 Joint Sealants

• Section 08 85 00 Glazing Sealants

• Section 32 13 73 Concrete Paving Joint Sealants

This document includes Specifier notes in hidden text. To view hidden text, www.bim.net/displaying-hidden-text-in-microsoft-word-step-by-step-instructions-for-windows-and-mac/

Paraseal®, TremPROOF®, ExoAir®, Spectrem®, Tremflex®, Dymeric®, and Proglaze® are registered trademarks of Tremco, Inc.

SECUREROCK® is a registered trademark of United States Gypsum Company.

This document is Copyright© 2014 by Tremco, Inc.

SECTION 07 18 00 – TRAFFIC COATINGS, VEHICULAR TRAFFIC

PART 1 – GENERAL

SECTION INCLUDES

Polyurethane and epoxy traffic coatings for vehicular traffic applications.

Pavement markings.

RELATED REQUIREMENTS

Specifier: If retaining this optional Related Requirements Article, edit to include only those sections included in project manual.

Section 03 31 00 "Cast-in-Place Concrete" for moisture curing of concrete traffic coating substrate.

Sections 07 90 00 / 07 95 00 Joint Protection/ Expansion Control

Section 07 92 00 "Joint Sealants" for joint sealants and accessories and joint preparation.

REFERENCES

Specifier: If retaining this optional References Article, edit to include only those references cited in the edited section.

References, General: Versions of the following standards current as of the date of issue of the project apply to the Work of this Section.

ASTM International (ASTM): www.astm.org:

ASTM C920 - Standard Specification for Elastomeric Joint Sealants

ASTM C957 - Standard Specification for High-Solids Content, Cold Liquid-Applied Elastomeric Waterproofing Membrane With Integral Wearing Surface

ASTM C1127 - Standard Guide for Use of High Solids Content, Cold Liquid-Applied Elastomeric Waterproofing Membrane with an Integral Wearing Surface

ASTM C1193 - Standard Guide for Use of Joint Sealants

ASTM D4258 - Standard Practice for Surface Cleaning Concrete for Coating

ASTM D4259 - Standard Practice for Abrading Concrete

ASTM E1980 - Standard Practice for Calculating Solar Reflectance Index of Horizontal and Low-Sloped Opaque Surfaces

International Concrete Repair Institute (ICRI): www.icri.org:

ICRI 310.2R - Selecting and Specifying Concrete Surface Preparation for Sealers, Coatings, Polymer Overlays, and Concrete Repair

UL Laboratories, Inc.(UL): www.ul.com:

UL790 - Standard Test Methods for Fire Tests of Roof Coverings

U. S. Environmental Protection Agency (EPA): www.epa.gov:

40 CFR 59, Subpart D - National Volatile Organic Compound Emission Standards for Architectural Coatings

ADMINISTRATIVE REQUIREMENTS

Preinstallation Conference: Conduct conference at Project Site.

Review requirements for traffic coating products and installation, including surface preparation, substrate conditions, expansion joint requirements, project and manufacturer's details, installation procedures, mockups, testing and inspection requirements, protection and repairs, and coordination and sequencing of traffic coating work with work of other Sections.

ACTION SUBMITTALS

Product Data: For each type of traffic coating product and expansion joint accessory specified, indicating compliance with requirements.

Shop Drawings: Show locations for traffic coating system components. Show details for each type of substrate, movement joints, corners, and edge conditions, including penetrations, transitions, and terminations.

INFORMATIONAL SUBMITTALS

Qualification Data: For Installer, manufacturer[, and traffic coating Inspector].

Certification of manufacturer's approval of Installer.

Product Test Reports: Test data for traffic coating products and traffic coating system, by qualified testing agency, indicating proposed traffic coating meets performance requirements, when requested by Architect.

Warranty: Sample of unexecuted manufacturer and installer special warranties.

Field quality control reports.

QUALITY ASSURANCE

Installer Qualifications: A manufacturer-approved firm with minimum [five] years' experience in installation of specified products in successful use on similar projects, employing workers trained by manufacturer, including a full-time on-site supervisor with a minimum of [three] years' experience installing similar work, and able to communicate verbally with Contractor[, Architect,] and employees.

Manufacturer Qualifications: A qualified manufacturer [listed in this Section] with minimum five years experience in manufacture of traffic coating as one of its principal products.

Manufacturer's product submitted has been in satisfactory operation on five similar installations for at least five years.

Specifier: Retain "Approval of Manufacturers and Comparable Products" Paragraph below to provide control over qualifying of substituted manufacturers.

Approval of Manufacturers and Comparable Products: [Submit] [Prime Bidder must submit] the following in accordance with project substitution requirements, within time allowed for substitution review:

Completed and signed Substitution Request form.

Product data, including certified independent test data indicating compliance with requirements.

Sample shop drawings from similar project.

Project references: Minimum of five installations of similar system not less than five years old, with Owner and Architect contact information.

Name and resume of proposed qualified Inspector.

Sample warranty.

Specifier: Retain "Testing Agency Qualifications" Paragraph if Contractor is required to provide independent inspections under Part 3 Field Quality Control article.

Testing Agency Qualifications: Qualified independent agency experienced in the installation of the specified traffic coating system, and qualified to perform observation and inspection specified in Field Quality Control Article to determine Installer’s compliance with the requirements of this Project, acceptable to Architect, retained by the Contractor.

Mockups: Provide traffic coating mockup application within mockups required in other sections, or if not specified, in an area of not less than 150 sq. ft. (14 sq. m) of surface where directed by [Architect] [Owner] for each type of substrate condition. Include examples of surface preparation, crack and joint treatment, traffic coating application, slip-resistant aggregate application, and flashing, transition, and termination conditions, to set quality standards for execution.

Include intersections of deck traffic coating with adjacent vertical coating and moisture control system applications.

If applicable, include no less than 13 ft (3.96m) including a minimum of one splice joint of  Willseal Expansion Joint System.

DELIVERY, STORAGE AND HANDLING

Accept materials on site in manufacturer's unopened original packaging.

Store products in weather protected environment, clear of ground and moisture, within temperature ranges recommended by traffic coating manufacturer.

Specifier: Retain first option in "Construction Waste" Paragraph below for LEED projects; retain second option for other projects.

Construction Waste: Store and dispose of packaging materials and construction waste in accordance with requirements of Division 01 Section ["Construction Waste Management"] ["Temporary Facilities and Controls."]

ENVIRONMENTAL REQUIREMENTS

Environmental Limitations: Apply traffic coating within the range of ambient and substrate temperatures recommended by traffic coating manufacturer.

Protect substrates from environmental conditions that affect coating performance.

Do not apply traffic coating (or expansion joint accessories if applicable), to a damp or wet substrate or during snow, rain, fog, or mist or when dew is present.

SCHEDULING

Coordinate installation of traffic coating with completion of roofing and other work requiring interface with traffic coating.

Schedule work so traffic coating system (including expansion joints if applicable) applications may be inspected prior to concealment.

WARRANTY

Specifier: Consult Tremco representative for available special project warranty terms and conditions.

Special Manufacturer's Warranty: Manufacturer's standard form in which traffic coating manufacturer agrees to furnish and traffic coating material to repair or replace those materials installed according to manufacturer's written instructions that exhibit material defects or otherwise fail to perform as specified under normal use within warranty period specified.

Access for Repair: Owner shall provide unimpeded access to the Project and the traffic coating system for purposes of testing, leak investigation, and repair, and shall reinstall removed cladding and overburden materials upon completion of repair.

Cost Limitation: Manufacturer's obligation for repair or replacement shall be limited to the original installed cost of the work.

Warranty Period: [ ] years date of Substantial Completion.

Special warranties specified in this article exclude deterioration or failure of traffic coating materials from the following:

Movement of the structure caused by structural settlement or stresses on the traffic coating exceeding manufacturer's written specifications for elongation.

Mechanical damage caused by outside agents.

- PRODUCTS

MANUFACTURERS

Specifier: Retain and edit option below if substitutions are allowed for project.

Basis-of-Design Products: Provide traffic coating products manufactured by Tremco Incorporated, Commercial Sealants & Waterproofing Division, Beachwood OH; (866) 321-6357; email: techresources@tremcoinc.com; www.tremcosealants.com, [or comparable products of other manufacturer approved by Architect in accordance with Instructions to Bidders and Division 01 General Requirements].

Source Limitations: Provide traffic coating system materials and accessory products from single-source manufacturer.

PERFORMANCE REQUIREMENTS

General: Traffic coating system shall be capable of performing as a continuous watertight installation and as a moisture drainage plane transitioned to adjacent flashings and discharging water to the structure exterior. Traffic coating shall accommodate normal substrate movement and seal expansion and control joints, construction material transitions, opening transitions, penetrations, and perimeter conditions without resultant moisture deterioration.

Compatibility: Provide traffic coating system materials that are compatible with one another and with adjacent materials under conditions of service and application required, as demonstrated by traffic coating manufacturer based on testing and field experience.

Specifier: Retain "Fire-Test-Response Characteristics" Paragraph below for traffic coatings that are roof coverings; revise to suit Project.  Indicate class for each traffic coating when tested according to ASTM E 108.  Verify requirements of authorities having jurisdiction and of Owner's insurer.

Fire-Test-Response Characteristics:  Provide traffic-coating materials with the fire-test-response characteristics as determined by testing identical products per test method below for deck type and slopes indicated by an independent testing and inspecting agency that is acceptable to authorities having jurisdiction.

[Class A] covering per UL790[ or ASTM E108).

Specifier Retain "Energy Performance" paragraph below to specify cool-roof performance for traffic coatings that are installed on exposed decks.  Verify availability of compliant products with manufacturers..

Specifier: Retain "VOC Content" Paragraph below if required for LEED-NC or LEED-CS Credit IEQ 4.2 and product is used inside the weatherproofing system such as for equipment rooms and underground parking.  Coordinate product selection to ensure that products comply.

Paragraph below may apply to limitations on VOCs of authorities having jurisdiction; verify local requirements..

Specifier:  Vulkem 950NF Topcoat can be used both as an intermediate coat for the Tremco heavy duty system and a topcoat for interior applications. Vulkem 951NF is a low-odor topcoat designed for exterior applications and for use over Vulkem 950NF in heavy duty applications.

TRAFFIC COATING FOR VEHICULAR TRAFFIC, EXTERIOR EXPOSURE

Traffic Coating:  Manufacturer's standard low-odor, low-VOC, exterior exposure, traffic-bearing, seamless, high-solids-content, cold liquid-applied, elastomeric, waterproofing membrane system with integral wearing surface for vehicular traffic; meeting ASTM C957 and below 100 g/L maximum per 40 CFR 59, Subpart D (EPA Method 24) and complying with requirements of authorities having jurisdiction

Basis of Design Products: Tremco, Inc., Vulkem 350NF[360NF]/Vulkem Epoxy WC/951NF[950NF]

Primer:  Liquid primer recommended for substrate and conditions by traffic-coating manufacturer. When required.

Base Coat:  Aromatic polyurethane.

Wear Coat: Aromatic epoxy.

- 20 to 40 mesh silica sand broadcast to refusal

Top Coat:  Aliphatic polyurethane with UV inhibitors [Aromatic polyurethane for interior use]

- Color:  As selected by Architect from manufacturer's full range.

Traffic Coating: Manufacturer's standard low-odor, low-VOC, exterior exposure, traffic-bearing, seamless, high-solids-content, cold liquid-applied, elastomeric, waterproofing membrane system with integral wearing surface for vehicular traffic; meeting ASTM C957 and below 350 g/L maximum per 40 CFR 59, Subpart D (EPA Method 24) and complying with requirements of authorities having jurisdiction

          - Basis of Design Products: Tremco, Inc., Vulkem 350NF[350]/Vulkem Epoxy WC/346TC

Primer:  Liquid primer recommended for substrate and conditions by traffic-coating manufacturer. When required.

Base Coat:  Aromatic polyurethane.

Wear Coat: Aromatic epoxy.

- 20 to 40 mesh silica sand broadcast to refusal

Top Coat:  Aliphatic polyurethane with UV inhibitors.

- Color:  As selected by Architect from manufacturer's full range.

ACCESSORY MATERIALS

General: Accessory materials as described in manufacturer's written installation instructions, recommended to produce complete traffic coating system meeting performance requirements, and compatible with traffic coating material and adjacent materials.

Single-Component, Non-Sagging Urethane Joint Sealant:  ASTM C920, Type NS, Class 50.

          - Basis of Design Product: Tremco Incorporated; Dymonic 100.

        - Expansion Joint Pre-compressed or Closed Cell, Monolithic Foam System. Foam Structure Must not Contain Unbonded Foam Laminations;

          - Willseal® 250 for use in both vehicular and pedestrian traffic applications.

          - Willseal® 250-R for use in both vehicular and pedestrian traffic applications requiring additional point load resistance.

          - Willseal® 250-CR for use in both vehicular and pedestrian traffic applications subject to chemical exposure, reference Willseal list of approved chemicals.

          - Willseal® Coreseal for use in both vehicular and pedestrian traffic applications requiring +/-25% movement capability, closed cell, and a lightweight seal.

          - Willseal® Color Coreseal – H for use in both vehicular and pedestrian traffic applications requiring +/-25% movement capability, closed cell, and a lightweight seal. For vertical applications refer to Color Coreseal – V.

          - Willseal® Color Coreseal – CM for use in both vehicular and pedestrian traffic applications requiring +/-25% movement capability, closed cell, and a lightweight seal with an integrated waterproofing membrane. For vertical applications refer to Color Coreseal – V.

          - Willseal® FR-H - for use in both vehicular and pedestrian traffic applications requiring hourly fire rated systems. For vertical applications refer to Willseal® FR-V.

          - Willseal® approved accessory sealants as per Willseal application instructions.

- EXECUTION

      - EXAMINATION

Surface Condition:  Before applying traffic coating materials, examine substrate and conditions to ensure substrates are fully cured and free from high spots, depressions, loose and foreign particles and other deterrents to adhesion, and conditions comply with manufacturer's written recommendations.

Verify concrete surfaces are visibly dry, have cured for time period recommended by traffic coating manufacturer, and are free from release agents, curing agents, laitance, and other contaminates.

Test surfaces following cleaning and abrasion specified below.

Test for capillary moisture by method recommended in writing by traffic-coating manufacturer.

Test for traffic coating adhesion per manufacturer's recommended method.

Notify Architect in writing of unsatisfactory conditions.

Proceed with installation once unsatisfactory conditions have been corrected.

PREPARATION

Surface Preparation: Clean, prepare, and treat substrates in accordance with ASTM C1127 and traffic coating manufacturer's written instructions.

Remove contaminants, curing compounds, and film-forming coatings from substrates.

Remove projections and excess materials and fill voids with manufacturer's recommended substrate patching material.

Mechanically abrade concrete surfaces to a uniform profile in accordance with ASTM D4259 and meeting ICRI Surface Profile CSP 2-4. Do not acid etch.

Clean prepared surfaces in accordance with ASTM D4258.

Protect adjacent finished surfaces by masking. Mask termination point on vertical surfaces. Protect weep holes and drains.

For accessory materials, follow manufacturers application instructions.

TERMINATIONS AND PENETRATIONS

Prepare vertical and horizontal surfaces at horizontal to vertical transitions, terminations, joints, and penetrations through traffic coatings in accordance with ASTM C1127 and manufacturer's written instructions, using accessory materials specified.

At terminations of traffic coating exposed to traffic, rout 1/4 by 1/4 inch keyway in concrete.

Detail Preparation: Prepare non-moving shrinkage cracks, large cracks, construction joints, expansion joints, projections and protrusions, penetrations, drains, and changes in plane in accordance with manufacturer's written instructions and details, .

Prepare joints and cracks in substrate in accordance with ASTM C1127 and ASTM D4258 and manufacturer's written instructions.

Joint Sealant Installation: Comply with ASTM C1193 and manufacturer's written instructions. Allow joint sealants to cure adequately before coating with traffic coating.

Provide joint sealant cants with backer rods at penetrations and at horizontal-to-vertical intersections. Tool sealant material to form 45 degree angle transition.

Rout and fill cracks with joint sealant and tool flush with surface.

Feather edges of joint sealant applications.

Allow joint sealant to cure. Apply detail coat of base coat over unfilled and filled cracks and joints, and feather terminating edges.

Fill expansion joints less than 1” with backer rod and joint sealant. Do not apply traffic coating over expansion joints.

Fill expansion joints greater than 1” with specified Willseal® Expansion Joint material, contact Tremco for sealant recommendation. Do not apply traffic coating over expansion joints

VEHICULAR TRAFFIC-COATING APPLICATION

Primer: Prime metal surfaces, porous surfaces (when required), and preceding coats left uncoated for more than 24 hours or that have lost surface tack, with manufacturer's recommended primer. Allow to cure before proceeding.

Specifier: Retain first paragraph below if applicable; recommended for large scale or critical applications.

Start traffic-coating application in presence of manufacturer's technical representative.

Apply traffic coating according to ASTM C1127 and manufacturer's written instructions.

Verify that wet film thickness of each coat complies with requirements every [100 sq. ft. (9 sq. m)].

Specifier: Edit number of coats below based upon manufacturer's recommendation for type of traffic. Intermediate coat is typically recommended for heavy traffic areas.

Apply number of coats of specified compositions for vehicular traffic coating at locations indicated on Drawings.

Base Coat: Single application of not less than 25 wet mils (0.635 mm).

Intermediate Coat: Single application at not less than 12 wet mils (.0305 mm). Immediately broadcast aggregate into wet coat.

Top Coat: Single application at not less than 12 wet mils (.0305).

Apply traffic coatings to prepared wall terminations and vertical surfaces to height indicated; omit aggregate on vertical surfaces.

Cure traffic coatings.  Prevent contamination and damage during application and curing stages.

FIELD QUALITY CONTROL

Specifier: Retain "Testing Agency" Paragraph below if applicable to Project, and edit to identify party retaining independent agency to perform tests and inspections.

Testing Agency: [Owner will engage] [Engage] a qualified testing agency to inspect substrate conditions, surface preparation, traffic coating application, protection, and drainage components, and to furnish reports to Architect.

Coordination of Testing: Cooperate with testing agency.  Allow access to work areas and staging.  Notify testing agency in writing of schedule for Work of this Section to allow sufficient time for testing and inspection.

Do not cover Work until testing and inspection is completed and accepted.

Reporting:  Forward written inspection reports to the Architect within 3 working days of the inspection and test being performed.

Correction:  Correct deficient applications not passing tests and inspections, make necessary repairs, and retest as required to demonstrate compliance with requirements.

CLEANING AND PROTECTING

Clean spills, stains, and overspray resulting application utilizing cleaning agents recommended by manufacturers of affected construction.  Remove masking materials.

Protect traffic coating from damage from subsequent work.  Protect traffic coating materials from exposure to UV light for period in excess of that acceptable to traffic coating manufacturer; replace overexposed materials and retest.

END OF SECTION